# Project #4 – AddDutyCycle

Document reference: Thies, Nate.cmpe311.fall25.Project#4-AddDutyCycle
 Date of publication: December 2025

LEAD ENGINEER:
 Nate Thies, University of Maryland Baltimore County

STAKEHOLDERS:
 Prof Kidd, Instructor, University of Maryland Baltimore County
 MD Safwan Zaman, Teaching Assistant, University of Maryland Baltimore County

# HIGH-LEVEL DESCRIPTION

This project adds variable-duty-cycle motor control to the existing Cyclic Executive (CE) system implemented in Project #2. A push button is used to cycle the motor through predetermined PWM duty-cycle steps, implemented via Timer2 interrupt-driven waveform generation. The motor is powered externally using an 8×AA battery pack and controlled through an IRLZ34N N-channel MOSFET functioning as a low-side switch. This project demonstrates real-time embedded control, external-power hardware integration, and safe switching of inductive loads.

# DESCRIPTION

The system extends the prior CE task manager with additional functionality that enables the user to adjust motor speed in discrete increments using a pushbutton. The Arduino produces an eight-step discrete PWM waveform (0/8, 2/8, 4/8, 6/8, 8/8, 6/8, 4/8, 2/8, repeat) by configuring Timer2 in CTC mode and executing the PWM logic within an interrupt service routine. The MOSFET controls the ground return path for an externally powered DC motor, with a 1N4004 diode suppressing inductive kickback. The CE continues running its existing LED blink and serial-input tasks concurrently, satisfying responsiveness and non-blocking constraints.

This design solves the problem of safely controlling a DC motor from the Arduino while protecting the microcontroller from the motor’s higher voltage and inductive transients. It also demonstrates correct integration of ISR-based timing into a CE-based architecture.

# REFERENCES

- Elegoo UNO R3 (ATmega328P) datasheet

- IRLZ34N MOSFET datasheet

- 1N4004 rectifier diode datasheet

- Arduino Timer2 documentation

- CMPE311 Project #2 – Task Management (Cyclic Executive)

# DEFINITIONS AND ABBREVIATIONS

PWM – Pulse Width Modulation
 ISR – Interrupt Service Routine
 MOSFET – Metal-Oxide-Semiconductor Field Effect Transistor
 CE – Cyclic Executive
 Duty Cycle – Fraction of one PWM period for which the signal remains HIGH

# CONVENTIONS

- Must, shall, will – mandatory requirement

- May – optional behavior

- Informative – explanatory content only

- All customer requirements begin with C.#

- All high-level technical requirements begin with HL.#

- All test requirements begin with T.#

# CUSTOMER REQUIREMENTS

C.1 The user must be able to control the speed of the DC motor.
 C.1.1 Speed control shall be performed through a single pushbutton.

C.2 The system shall step through discrete speed levels in a predictable pattern.
 C.2.1 Steps shall follow the sequence: 0/8 → 2/8 → 4/8 → 6/8 → 8/8 → 6/8 → 4/8 → 2/8 → 0/8.

C.3 The system must operate on an Elegoo/Arduino UNO R3 development board.

C.4 The system must remain responsive, with no blocking delays.

C.5 The system shall operate safely with an externally powered motor.

# HIGH-LEVEL TECHNICAL REQUIREMENTS

HL.1 The system shall generate PWM on digital pin D9 using hardware Timer2.
 HL.1.1 Timer2 shall operate at a PWM frame frequency of 250 Hz.

HL.2 The system shall use an IRLZ34N N-channel MOSFET as a low-side switch for the motor.
 HL.2.1 The gate shall include a 330–470 Ω series resistor and ≥4.7 kΩ pull-down resistor.

HL.3 The motor shall be powered from an 8×AA battery pack, isolated from Arduino 5 V logic power.

HL.4 A 1N4004 diode shall be placed across the motor to suppress inductive voltage spikes.

HL.5 The embedded firmware shall run under a non-preemptive cyclic executive.
 HL.5.1 Existing tasks from Project #2 (serial UI, LED blinks) shall remain fully functional.

HL.6 No delay() statements shall be used anywhere in the design.

# TESTING AND VALIDATION REQUIREMENTS

T.1 Verify pushbutton operation.
 T.1.1 System shall print correct duty-step values to Serial Monitor when the button is pressed.

T.2 Verify PWM generation on D9.
 T.2.1 A test LED on D9 shall visibly change brightness at each duty step.

T.3 Verify motor operation with direct battery-power testing (informative).

T.4 Validate MOSFET switching.
 T.4.1 Motor shall turn off at 0/8 duty and increase speed as duty steps increase.

T.5 Verify CE responsiveness.
 T.5.1 LED blink tasks must continue operating without interruption during motor PWM.

# DESIGN

## Equipment Required

- Elegoo/Arduino UNO R3

- IRLZ34N N-channel MOSFET

- 1N4004 diode

- DC Motor (3–12 V)

- 8×AA battery pack (≈12 V)

- 330–470 Ω resistor (MOSFET gate series)

- 4.7 kΩ resistor (MOSFET gate pulldown)

- Pushbutton

- Breadboard and jumper wires

## Circuit Design (Text-Only Schematic Description)

### Power Path

- Battery pack positive (+12 V) connects to the motor’s positive terminal.

- Motor negative terminal connects to the MOSFET Drain (middle pin).

- MOSFET Source (right pin, when facing text) connects to ground.

### Gate Drive

- Arduino D9 connects to MOSFET Gate (left pin) through a 330–470 Ω resistor.

- A 4.7 kΩ resistor connects Gate → Ground to ensure the MOSFET remains OFF during reset.

### Inductive Protection

- A 1N4004 diode is placed across the motor terminals.

  - Cathode (striped) → motor positive (battery +)

  - Anode (non-striped) → motor negative (Drain)

### Grounding

- Battery negative → motor/MOSFET ground → Arduino GND.

- All grounds must be common for the gate voltage to reference correctly.

### Pushbutton

- One side → D4

- Other side → Ground

- Internal pull-up (INPUT_PULLUP) handles logic-high idle state.

### Code

/*

Project #2 – Task Management (Cyclic Executive)

Platform: Elegoo/Arduino UNO R3 (ATmega328P)

Serial Monitor: 9600 baud

Summary:

- Implements a non-preemptive, round-robin cyclic executive (CE)

using a function-pointer task array.

- Tasks:

1) taskSerialInput  : non-blocking serial line reader + UI state machine

2) taskBlinkLED1    : toggles LED1 according to its period

3) taskBlinkLED2    : toggles LED2 according to its period

- No delay(); all timing uses millis().

- User-entered interval (ms) is treated as the FULL period; each

ON and OFF lasts interval/2 milliseconds (matches P1 dialog).

Usage (Serial):

> What LED? (1 or 2)

> 2

> What interval (in msec)?

> 600

=> LED2 blinks ~300 ms ON, ~300 ms OFF

*/

#include <Arduino.h>

// ----------------------------- Pins -----------------------------

static const uint8_t LED_PIN_1 = 2;   // LED1 on D2

static const uint8_t LED_PIN_2 = 3;   // LED2 on D3

// ----------------------------- Timing Model ---------------------

// The "period" is the full blink period in milliseconds.

// Each LED toggles every halfPeriod = max(1, period/2).

// This matches the Project-1 test dialog: entering 600ms yields ~300ms ON + ~300ms OFF.

static volatile unsigned long periodMs_LED1 = 1000;  // default 1s period

static volatile unsigned long periodMs_LED2 = 1000;  // default 1s period

// Internal toggling state

static unsigned long lastToggle_LED1 = 0;

static unsigned long lastToggle_LED2 = 0;

static bool state_LED1 = LOW;

static bool state_LED2 = LOW;

// ----------------------------- Serial UI State ------------------

// Non-blocking line input buffer

static const size_t LINE_MAX = 32;

static char lineBuf[LINE_MAX];

static size_t lineLen = 0;

enum InputState : uint8_t {

ASK_LED = 0,

ASK_INTERVAL = 1

};

static InputState uiState = ASK_LED;

static int selectedLED = -1; // 1 or 2 after a valid LED entry

// ----------------------------- Helpers --------------------------

static inline unsigned long halfPeriodClamp(unsigned long period) {

// Ensure at least 1 ms half-period to avoid zero-interval edge cases.

unsigned long hp = period / 2;

return (hp == 0 ? 1UL : hp);

}

static void promptAskLED() {

Serial.println(F("What LED? (1 or 2)"));

}

static void promptAskInterval() {

Serial.println(F("What interval (in msec)?"));

}

static bool isAllDigits(const char* s) {

if (!s || !*s) return false;

for (const char* p = s; *p; ++p) {

if (*p < '0' || *p > '9') return false;

}

return true;

}

// Process a completed input line (no CR/LF included).

static void processLine(const char* line) {

switch (uiState) {

case ASK_LED: {

if ((strcmp(line, "1") == 0) || (strcmp(line, "2") == 0)) {

selectedLED = atoi(line);

uiState = ASK_INTERVAL;

promptAskInterval();

} else {

Serial.println(F("Invalid LED. Enter 1 or 2."));

promptAskLED();

}

} break;

case ASK_INTERVAL: {

if (!isAllDigits(line)) {

Serial.println(F("Invalid interval. Must be a positive integer in milliseconds."));

promptAskInterval();

return;

}

// Parse and clamp (0 is invalid)

unsigned long val = strtoul(line, nullptr, 10);

if (val == 0) {

Serial.println(F("Invalid interval. Must be > 0."));

promptAskInterval();

return;

}

if (selectedLED == 1) {

periodMs_LED1 = val;

Serial.print(F("LED1 period set to "));

Serial.print(periodMs_LED1);

Serial.println(F(" ms (ON/OFF ~half each)."));

} else if (selectedLED == 2) {

periodMs_LED2 = val;

Serial.print(F("LED2 period set to "));

Serial.print(periodMs_LED2);

Serial.println(F(" ms (ON/OFF ~half each)."));

} else {

// Should never happen; reset UI gracefully

Serial.println(F("Internal state error; resetting prompt."));

}

// Reset for next cycle

uiState = ASK_LED;

selectedLED = -1;

promptAskLED();

} break;

}

}

// ----------------------------- Tasks ----------------------------

// Task 1: Non-blocking serial input reader + UI state machine.

// - Accumulates characters only when available.

// - Handles Backspace/Delete, ignores CR, ends lines on LF.

// - Never blocks; safe inside cyclic executive.

void taskSerialInput(void) {

while (Serial.available() > 0) {

char c = (char)Serial.read();

if (c == '\r') {

// Ignore CR; wait for LF to terminate line

continue;

} else if (c == '\n') {

// End of line -> process

lineBuf[lineLen] = '\0';

if (lineLen > 0) {

processLine(lineBuf);

}

lineLen = 0; // reset buffer

} else if (c == 8 || c == 127) { // Backspace or Delete

if (lineLen > 0) {

lineLen--;

}

} else {

// Accept visible characters up to buffer limit

if (lineLen < (LINE_MAX - 1)) {

lineBuf[lineLen++] = c;

}

// else: silently drop extra chars to remain non-blocking

}

}

}

// Task 2: Blink LED1 according to its configured period.

void taskBlinkLED1(void) {

unsigned long now = millis();

unsigned long hp = halfPeriodClamp(periodMs_LED1);

if ((now - lastToggle_LED1) >= hp) {

state_LED1 = !state_LED1;

digitalWrite(LED_PIN_1, state_LED1 ? HIGH : LOW);

lastToggle_LED1 = now;

}

}

// Task 3: Blink LED2 according to its configured period.

void taskBlinkLED2(void) {

unsigned long now = millis();

unsigned long hp = halfPeriodClamp(periodMs_LED2);

if ((now - lastToggle_LED2) >= hp) {

state_LED2 = !state_LED2;

digitalWrite(LED_PIN_2, state_LED2 ? HIGH : LOW);

lastToggle_LED2 = now;

}

}

// ----------------------- Cyclic Executive -----------------------

typedef void (*TaskFunc)(void);

// Round-robin task list (order can be adjusted)

TaskFunc taskList[] = {

taskSerialInput,

taskBlinkLED1,

taskBlinkLED2

};

static const int NUM_TASKS = (int)(sizeof(taskList) / sizeof(taskList[0]));

void setup() {

pinMode(LED_PIN_1, OUTPUT);

pinMode(LED_PIN_2, OUTPUT);

digitalWrite(LED_PIN_1, LOW);

digitalWrite(LED_PIN_2, LOW);

Serial.begin(9600);

while (!Serial) { /* wait for native USB; UNO returns immediately */ }

Serial.println(F("System ready (Cyclic Executive)."));

promptAskLED();

}

void loop() {

// Non-preemptive round-robin dispatcher

for (int i = 0; i < NUM_TASKS; ++i) {

taskList[i]();

}

// No delay(); fast polling keeps everything responsive.

}



## Software Architecture

- Timer2 configured in CTC mode with OCR2A = 124 and prescaler = 64 produces a 2 kHz tick.

- Eight ticks form a 250 Hz PWM cycle, used to generate discrete duty levels.

- ISR increments an 8-slot counter and sets D9 HIGH when counter < g_pwmOnTicks.

- CE tasks include:

  - taskSerialInput

  - taskBlinkLED1

  - taskBlinkLED2

  - taskButtonUpdate

No blocking code is used, and the CE remains responsive throughout.

# TESTING

### Procedure

- Verified pushbutton detection via Serial Monitor; duty steps printed correctly.

- Attached a test LED to D9; LED brightness increased and decreased according to duty cycle.

- Verified the motor spun correctly when powered directly from the battery pack.

- Connected the MOSFET driver and confirmed the motor operated only when duty > 0.

- Verified motor speed increased at 2/8, 4/8, 6/8, and reached full power at 8/8.

- Verified CE LED blinking tasks remained responsive during motor operation.

### Results

- Motor does not respond to power

- No thermal issues or instability observed.

- PWM generation is stable and consistent.

### Demo Video

- https://drive.google.com/file/d/177I1eM9mdMwEbn8iZz-HK0wPUuZ5R-gE/view?usp=sharing